Project title:

Generate QR codes with Arduino on OLED display

Creator:

Mario’s Ideas

Source:

https://projecthub.arduino.cc/ and

https://www.youtube.com/watch?v=sSSpmnxdzPM

Tag:

Arduino Nano, Arduino IDE.

Project Description:

Generate QR codes and display them on tiny OLED display. There are two methods to generate the QR code:

- hardcode any given text (or URL) and

- enter the given text (URL) with keyboard

Codes for both methods are provided in this tutorial.

QR-code-with-Arduino.png

Components:

- Arduino Nano microcontroller board

- OLED SPI Display 128x64

Circuit Diagram:

3D Print (.stl) files:

Not applicable

Code (.ino C++):

- Generate QR code for a given text

here

#include <Wire.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

#include <qrcode.h>

// OLED display dimensions

#define OLED_WIDTH 128

#define OLED_HEIGHT 64

// OLED display object

Adafruit_SSD1306 display(OLED_WIDTH, OLED_HEIGHT, &Wire, -1);

void setup() {

// Initialize the OLED display

display.begin(SSD1306_SWITCHCAPVCC, 0x3C);

display.clearDisplay();

// Display the QR code for "Marios Id"

generateQRCode("GR8 that U R here:)");

}

void loop() {

// Nothing to do here

}

void generateQRCode(const char* text) {

// Create a QR code object

QRCode qrcode;

// Define the size of the QR code (1-40, higher means bigger size)

uint8_t qrcodeData[qrcode_getBufferSize(3)];

qrcode_initText(&qrcode, qrcodeData, 3, 0, text);

// Clear the display

display.clearDisplay();

// Calculate the scale factor

int scale = min(OLED_WIDTH / qrcode.size, OLED_HEIGHT / qrcode.size);

// Calculate horizontal shift

int shiftX = (OLED_WIDTH - qrcode.size*scale)/2;

// Calculate horizontal shift

int shiftY = (OLED_HEIGHT - qrcode.size*scale)/2;

// Draw the QR code on the display

for (uint8_t y = 0; y < qrcode.size; y++) {

for (uint8_t x = 0; x < qrcode.size; x++) {

if (qrcode_getModule(&qrcode, x, y)) {

display.fillRect(shiftX+x * scale, shiftY + y*scale, scale, scale, WHITE);

}

}

}

// Update the display

display.display();

}

Code (.ino C++):

- QR code for text entered using keyboard module

here

#include <Wire.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

#include <qrcode.h>

// OLED display dimensions

#define OLED_WIDTH 128

#define OLED_HEIGHT 64

// OLED display object

Adafruit_SSD1306 display(OLED_WIDTH, OLED_HEIGHT, &Wire, -1);

void setup() {

// Initialize the OLED display

display.begin(SSD1306_SWITCHCAPVCC, 0x3C);

display.clearDisplay();

// Display the QR code for "Marios Id"

generateQRCode("GR8 that U R here:)");

}

void loop() {

// Nothing to do here

}

void generateQRCode(const char* text) {

// Create a QR code object

QRCode qrcode;

#include <Wire.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

#include <qrcode.h>

// OLED display dimensions

#define OLED_WIDTH 128

#define OLED_HEIGHT 64

// OLED display object

Adafruit_SSD1306 display(OLED_WIDTH, OLED_HEIGHT, &Wire, -1);

//Potentiometer PIN A1

int Keyboard=A1;

// Variables capturing current and newly calculated position on the letter board (9x3 - 27 postions)

int New_X=0;

int Old_X=0;

int New_Y=0;

int Old_Y=0;

// Variable capturing output from Keyboard pin (Values 0 1023)

int Key_read=0;

int Prev_Key_read=1023;

boolean Key_pressed=false;

// String variable holding the text to transmit

String To_Transmit="";

char QR_data[20];

// Used for displaying Leter board

char Letters[3][9]={"ABCDEFGHI",

"JKLMNOPQR",

"STUVWXYZ_"};

void setup() {

// Initialize the OLED display

display.begin(SSD1306_SWITCHCAPVCC, 0x3C);

display.clearDisplay();

// Clear the buffer

display.clearDisplay();

display.display();

// Display filled in rect in the top section of the display when To_Transfer would be output

display.fillRect(0, 0, 128, 15, SSD1306_INVERSE);

display.drawRect(110, 2, 16, 12, SSD1306_BLACK);

display.setTextSize(1);

display.setTextColor(SSD1306_BLACK);

display.setCursor(113,4);

display.println("OK");

display.display();

// Display Letter Board 3 rows 9 character in each row

display.setTextSize(2);

display.setTextColor(SSD1306_WHITE);

for (int j=0; j<3;j++){

for (int i=0; i<9;i++){

display.setCursor(i*12+2*i+1,j*16+17);

display.println(Letters[j][i]);

delay(10);

display.display();

}

}

// Highlight character A by displaying Inverse rect at first position

display.fillRect(0, 16, 12, 16, SSD1306_INVERSE);

display.display();

// Display the QR code for "Marios Id"

//generateQRCode("GR8 that U R here:)");

}

void generateQRCode(const char* text) {

// Create a QR code object

QRCode qrcode;

// Define the size of the QR code (1-40, higher means bigger size)

uint8_t qrcodeData[qrcode_getBufferSize(3)];

qrcode_initText(&qrcode, qrcodeData, 3, 0, text);

// Clear the display

display.clearDisplay();

// Calculate the scale factor

int scale = min(OLED_WIDTH / qrcode.size, OLED_HEIGHT / qrcode.size);

// Calculate horizontal shift

int shiftX = (OLED_WIDTH - qrcode.size*scale)/2;

// Calculate horizontal shift

int shiftY = (OLED_HEIGHT - qrcode.size*scale)/2;

// Draw the QR code on the display

for (uint8_t y = 0; y < qrcode.size; y++) {

for (uint8_t x = 0; x < qrcode.size; x++) {

if (qrcode_getModule(&qrcode, x, y)) {

display.fillRect(shiftX+x * scale, shiftY + y*scale, scale, scale, WHITE);

}

}

}

// Update the display

display.display();

}

void Highlight_letter(int X, int X_Old, int Y, int Y_Old){

// When position changes

// Draw the inverse rect in the Old_pos to deactivate  the highlight in the old spot

// Draw the inverse rect to Highlite the new spot

// Displaying Inverse rect in a new position to highlight

display.fillRect(X*12+2*X, Y*16 +16, 12, 16, SSD1306_INVERSE);

// Displaying Inverse rect in the old positon to unhighlight

display.fillRect(X_Old*12+2*X_Old, Y_Old*16 +16, 12, 16, SSD1306_INVERSE);

display.display();

}

void loop() {

Key_read =analogRead(Keyboard);

if (Prev_Key_read>1000 and Key_read<1000){

Key_pressed=true;

if (Key_read<10 and Old_X>0) New_X=Old_X-1;

if (Key_read>160 and Key_read<170 and Old_X<9) New_X=Old_X+1;

if (Key_read>25 and Key_read<35 and Old_Y>-1) New_Y=Old_Y-1;

if (Key_read>80 and Key_read<90 and  Old_Y<2 ) New_Y=Old_Y+1;

if (Key_read>350 and Key_read<360) {

if (New_Y!=-1){

To_Transmit=To_Transmit + Letters[New_Y][New_X];

display.setTextSize(2);

display.setCursor(3,1);

display.setTextColor(BLACK     );

display.fillRect(0, 0, 100, 15, SSD1306_WHITE);

display.println(To_Transmit);

display.display();

}

else {

To_Transmit.toCharArray(QR_data,20);

generateQRCode(QR_data );

delay(25000);

To_Transmit="";

int New_X=0;

int Old_X=0;

int New_Y=0;

int Old_Y=0;

// Display filled in rect in the top section of the display when To_Transfer would be output

display.clearDisplay();

display.fillRect(0, 0, 128, 15, SSD1306_INVERSE);

display.drawRect(110, 2, 16, 12, SSD1306_BLACK);

display.setTextSize(1);

display.setTextColor(SSD1306_BLACK);

display.setCursor(113,4);

display.println("OK");

display.display();

// Display Letter Board 3 rows 9 character in each row

display.setTextSize(2);

display.setTextColor(SSD1306_WHITE);

for (int j=0; j<3;j++){

for (int i=0; i<9;i++){

display.setCursor(i*12+2*i+1,j*16+17);

display.println(Letters[j][i]);

delay(10);

display.display();

}

}

// Highlight character A by displaying Inverse rect at first position

display.fillRect(0, 16, 12, 16, SSD1306_INVERSE);

display.display();

}

}

if (New_Y==-1 and Old_Y==0){

display.fillRect(110, 2, 16, 12, SSD1306_INVERSE);

display.fillRect(Old_X*12+2*Old_X, Old_Y*16 +16, 12, 16, SSD1306_INVERSE);

}

if (New_Y==0 and Old_Y==-1){

display.fillRect(110, 2, 16, 12, SSD1306_INVERSE);

display.fillRect(New_X*12+2*New_X, New_Y*16 +16, 12, 16, SSD1306_INVERSE);

Prev_Key_read=Key_read;

Old_X=New_X;

Old_Y=New_Y;;

}

if ((Old_X!=New_X or Old_Y!=New_Y) and Old_Y!=-1 ){

if (New_Y!=-1 )Highlight_letter (New_X,Old_X,New_Y,Old_Y);

Old_X=New_X;

Old_Y=New_Y;

}

}

display.display();

Prev_Key_read=Key_read;

}