Project title:

Generate QR codes with Arduino on OLED display

Creator:

Ruis Santos

Source:

https://randomnerdtutorials.com/esp32-web-server-arduino-ide/

Tag:

Arduino Nano, Arduino IDE.

Project Description:

A standalone web server with an ESP32 that controls output (two LEDs) using the Arduino IDE programming environment. The web server is mobile responsive and can be accessed with any device that is a browser on the local network. We’ll show you how to create the web server and how the code works step-by-step.

ESP32-web-server.png

Components:

- ESP32 development board

- 2x 5mm LED

- 2x 330 Ohm resistor

Circuit Diagram:

3D Print (.stl) files:

Not applicable

Code (.ino C++):

- Generate QR code for a given text

here

/*********

Rui Santos

Complete project details at https://randomnerdtutorials.com

*********/

// Load Wi-Fi library

#include <WiFi.h>

// Replace with your network credentials

const char* ssid = "REPLACE_WITH_YOUR_SSID";

const char* password = "REPLACE_WITH_YOUR_PASSWORD";

// Set web server port number to 80

WiFiServer server(80);

// Variable to store the HTTP request

String header;

// Auxiliar variables to store the current output state

String output26State = "off";

String output27State = "off";

// Assign output variables to GPIO pins

const int output26 = 26;

const int output27 = 27;

// Current time

unsigned long currentTime = millis();

// Previous time

unsigned long previousTime = 0;

// Define timeout time in milliseconds (example: 2000ms = 2s)

const long timeoutTime = 2000;

void setup() {

Serial.begin(115200);

// Initialize the output variables as outputs

pinMode(output26, OUTPUT);

pinMode(output27, OUTPUT);

// Set outputs to LOW

digitalWrite(output26, LOW);

digitalWrite(output27, LOW);

// Connect to Wi-Fi network with SSID and password

Serial.print("Connecting to ");

Serial.println(ssid);

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

// Print local IP address and start web server

Serial.println("");

Serial.println("WiFi connected.");

Serial.println("IP address: ");

Serial.println(WiFi.localIP());

server.begin();

}

void loop(){

WiFiClient client = server.available();   // Listen for incoming clients

if (client) {                             // If a new client connects,

currentTime = millis();

previousTime = currentTime;

Serial.println("New Client.");          // print a message out in the serial port

String currentLine = "";                // make a String to hold incoming data from the client

while (client.connected() && currentTime - previousTime <= timeoutTime) {  // loop while the client's connected

currentTime = millis();

if (client.available()) {             // if there's bytes to read from the client,

char c = client.read();             // read a byte, then

Serial.write(c);                    // print it out the serial monitor

header += c;

if (c == '\n') {                    // if the byte is a newline character

// if the current line is blank, you got two newline characters in a row.

// that's the end of the client HTTP request, so send a response:

if (currentLine.length() == 0) {

// HTTP headers always start with a response code (e.g. HTTP/1.1 200 OK)

// and a content-type so the client knows what's coming, then a blank line:

client.println("HTTP/1.1 200 OK");

client.println("Content-type:text/html");

client.println("Connection: close");

client.println();

// turns the GPIOs on and off

if (header.indexOf("GET /26/on") >= 0) {

Serial.println("GPIO 26 on");

output26State = "on";

digitalWrite(output26, HIGH);

} else if (header.indexOf("GET /26/off") >= 0) {

Serial.println("GPIO 26 off");

output26State = "off";

digitalWrite(output26, LOW);

} else if (header.indexOf("GET /27/on") >= 0) {

Serial.println("GPIO 27 on");

output27State = "on";

digitalWrite(output27, HIGH);

} else if (header.indexOf("GET /27/off") >= 0) {

Serial.println("GPIO 27 off");

output27State = "off";

digitalWrite(output27, LOW);

}

// Display the HTML web page

client.println("<!DOCTYPE html><html>");

client.println("<head><meta name=\"viewport\" content=\"width=device-width, initial-scale=1\">");

client.println("<link rel=\"icon\" href=\"data:,\">");

// CSS to style the on/off buttons

// Feel free to change the background-color and font-size attributes to fit your preferences

client.println("<style>html { font-family: Helvetica; display: inline-block; margin: 0px auto; text-align: center;}");

client.println(".button { background-color: #4CAF50; border: none; color: white; padding: 16px 40px;");

client.println("text-decoration: none; font-size: 30px; margin: 2px; cursor: pointer;}");

client.println(".button2 {background-color: #555555;}</style></head>");

// Web Page Heading

client.println("<body><h1>ESP32 Web Server</h1>");

// Display current state, and ON/OFF buttons for GPIO 26

client.println("<p>GPIO 26 - State " + output26State + "</p>");

// If the output26State is off, it displays the ON button

if (output26State=="off") {

client.println("<p><a href=\"/26/on\"><button class=\"button\">ON</button></a></p>");

} else {

client.println("<p><a href=\"/26/off\"><button class=\"button button2\">OFF</button></a></p>");

}

// Display current state, and ON/OFF buttons for GPIO 27

client.println("<p>GPIO 27 - State " + output27State + "</p>");

// If the output27State is off, it displays the ON button

if (output27State=="off") {

client.println("<p><a href=\"/27/on\"><button class=\"button\">ON</button></a></p>");

} else {

client.println("<p><a href=\"/27/off\"><button class=\"button button2\">OFF</button></a></p>");

}

client.println("</body></html>");

// The HTTP response ends with another blank line

client.println();

// Break out of the while loop

break;

} else { // if you got a newline, then clear currentLine

currentLine = "";

}

} else if (c != '\r') {  // if you got anything else but a carriage return character,

currentLine += c;      // add it to the end of the currentLine

}

}

}

// Clear the header variable

header = "";

// Close the connection

client.stop();

Serial.println("Client disconnected.");

Serial.println("");

}

}