Project title:

Arduino UNO with BME280 Temperature, Humidity, Pressure sensor module

Tag:

Arduino UNO, Arduino IDE, C++, BME280 sensor module

Component List:

- Arduino UNO board

- BME280 sensor module

- Breadboard

- Jumper wires

Circuit Diagram

Image-1: Circuit Diagram

Library:

- Wire.h

- SPI.h

- Adafruit_sensor.h

- Adafruit_BME280.h

Instructions:

- Wire up BME280 module to Arduino UNO board as per the above circuit diagram

Take note that the example here uses Arduino UNO:

- pin A4 for SDA (I2C data line)

- pin A5 for SCK (I2C clock line)

- 5V and GND.

- Run Arduino IDE, install the above libraries and its respective dependencies (if required).

- Connect Arduino UNO board to USB port. In this example the board is connected as COM7 port.

- Copy the code below and save it to local file as .ino file.

Take note that in this example we set:

- the baud rate: 9600

- Upload the code to the Arduino UNO board.

- Once successfully uploaded without error, open the Serial Monitor. Ensure that it is set to baud rate 9600, NewLine and Autoscroll selected.

Code

/*  Complete Project Details https://randomnerdtutorials.com  */

#include <Wire.h>

#include <SPI.h>

#include <Adafruit_Sensor.h>

#include <Adafruit_BME280.h>

#define BME_SCK 13

#define BME_MISO 12

#define BME_MOSI 11

#define BME_CS 10

#define SEALEVELPRESSURE_HPA (1013.25)

Adafruit_BME280 bme; // I2C

unsigned long delayTime;

void setup() {

Serial.begin(9600);

Serial.println(F("BME280 test"));

bool status;

status = bme.begin();

if (!status) {

Serial.println("Could not find a valid BME280 sensor, check wiring!");

while (1);

}

Serial.println("-- Default Test --");

delayTime = 1000;

Serial.println();

}

void loop() {

printValues();

delay(delayTime);

}

void printValues() {

Serial.print("Temperature = ");

Serial.print(bme.readTemperature());

Serial.println(" *C");

// Convert temperature to Fahrenheit

/*Serial.print("Temperature = ");

Serial.print(1.8 * bme.readTemperature() + 32);

Serial.println(" *F");*/

Serial.print("Pressure = ");

Serial.print(bme.readPressure() / 100.0F);

Serial.println(" hPa");

Serial.print("Approx. Altitude = ");

Serial.print(bme.readAltitude(SEALEVELPRESSURE_HPA));

Serial.println(" m");

Serial.print("Humidity = ");

Serial.print(bme.readHumidity());

Serial.println(" %");

Serial.println();

}

Images

Image-2: BME280 sensor module

Image-3: Complete setup

Image-4: Results on Serial Monitor

Other build files:

None